%% MATLAB (R2021a) CODE TO CALCULATE FIXTURE P-VALUE FROM RAW DATA

% Import data

% INDICATE CORRECT FILE PATH

% 25 days data from lavatory use in the men’s restroom

Data = readtable("C:\FILEPATH\08192022_12082022_MarianSpenceCefeteria_MensRestroom_Lavatory_2.csv");

% 2 days data from lavatory use in the women’s restroom

% INDICATE CORRECT FILE PATH

Data = readtable("C:\FILEPATH\09132022_09142022_MarianSpenceCefeteria_WomensRestroom_Lavatory_2.csv");

% Convert data from Table into vectors

DateData = Data.Date- timeofday(Data.Date); % Delete time to get only date DDMMYY

%StartTime = timeofday(datetime(Data.EventStartTime, 'ConvertFrom', 'datenum'));  % Use if data imported from .xlsx

StartTime = timeofday(datetime(Data.EventStartTime));   % Use if data imported from .csv

%EndTime = timeofday(datetime(Data.EventEndTime, 'ConvertFrom', 'datenum'));      % Use if data imported from .xlsx

EndTime = timeofday(datetime(Data.EventEndTime));   % Use if data imported from .csv

%Duration = Data.Duration_sec;  % Actual measurement in 0.25 seconds fraction

Duration = seconds(EndTime -StartTime);  %Calculate duration of flow in full seconds

% Find length of study

StudyDates = unique(DateData);

StudyLength = length(StudyDates);  % Number of Unique dates

% Check weekdays and Weekends

DoW = isweekend(StudyDates (:));   % 1 is weekend

%Input Information

NumFix = 2; % ENTER Number of fixtures monitored

Window = 60;  % Fixed observation window (minutes)

WinSec = Window*60;  % convert Seconds

Intv = 3600*24/WinSec;  % Number of intervals per observation window

% Create 24-hour timeline

Time_TL = duration(0,0,1:86400)';

% Create Variables for the results

Res_Sumary = [];

ProbString = [];

PeakHour = [];

Pv_event = [];

Pv_Dur = [];

aaT = [];

aaP = [];

aaDate = [];

ProbTL = zeros(Intv,StudyLength);

PulseTL = zeros(Intv,StudyLength);

DurTL = zeros(Intv,StudyLength);

for i = 1:StudyLength   % Loop to calculate daily probability values

% Set i = SPECIFIC NUMBER for a specific date in "StudyDates"

if i==13 || i == 14 % Exclude data on Thanksgiving and next day (zero water use)

continue

else

DailyTL = zeros(86400,1);  % date, time, 1=on, 0=off,

Ind = find(DateData==StudyDates(i));  % Indices for single day data

% Extract start and end times for each day

SrtTdata = StartTime(Ind);     % Start time

EndTdata = EndTime(Ind);       % End time

Durdata = Duration(Ind);      %% Duration of flow

DurCheck = [];

% Populate the timeline with flow details in binary numbers

for j = 1:length(Ind)

% Find indice from start to stop time (-1 because start time

% included

TInd = find(and(Time_TL >= SrtTdata(j), Time_TL <= EndTdata(j)-duration(0,0,1)));

DurCheck = [DurCheck; length(TInd)];

DailyTL(TInd,1)= DailyTL(TInd,1)+1;  % +1 Binary indicator of busy fixture

end

% Create timeline

%DataTL(:,i)= DailyTL(:,1);

ProbTime=[];

% Calculate probability of use and Number of pulse for each Observation window

for ij = 1:Intv

astrt = Time_TL(((ij-1)*WinSec)+1);

astop = Time_TL(ij*WinSec);

aInd = find(and(Time_TL>=astrt, Time_TL<=astop)); % Indices for each pulse

ProbTime =[ProbTime; astop];

PVCal = sum(DailyTL(aInd,1))/(NumFix*WinSec);  % p-value

ProbTL(ij,i) = PVCal;     % Save p-value for each observation window on each study day

PulseCnt = find(and(SrtTdata>=astrt, SrtTdata<=astop));

% Duration of flow duration in observation window

DurTL(ij,i)=sum(DailyTL(aInd,1));

% Number of pulses during observation window

PulseTL(ij,i) = length(PulseCnt);

end

% Indices for each observation window with the maximum number of events

MaxEvent = find(PulseTL(:,i)==max(PulseTL(:,i)));

PeakHour =[PeakHour; MaxEvent(1)];  % Hour with maximum number of events

Daily_ProbTL = ProbTL(:,i);

Daily_PulseTL= PulseTL(:,i);

% Peak hour p-values

% If more than one hour with equal maximum number of events , pick the first hour

Pv_event = [Pv_event;  ProbTL(MaxEvent(1),i)];

%Fixed the active window between 10am and 10pm (When the business

%is open to the public)

ActiveWindow = and(ProbTime >= duration(10,0,0) ,ProbTime < duration(22,0,0));

MontTime = find(ActiveWindow); %Indices for active times

% Summarize probability values for the time interval

aa = ProbTL(MontTime,i); %P-value Within Active window

aaT = [aaT; ProbTime(MontTime)];

aaP = [aaP; ProbTL(MontTime,i)];

aaDate=[aaDate; repmat(StudyDates(i),length(MontTime),1)];

% Summary of daily p-values and duration of pulses

%[ mean, maximum, std deviation and Sample size]

Res_Sumary = [Res_Sumary;  mean(aa) max(aa) std(aa)  length(aa)...

min(Durdata) mean(Durdata) std(Durdata) max(Durdata) length(Durdata)];

end

end

% Plot for peak hour probability over study period and

figure

% plot for daily peak hour p-values

subplot(2,1,1)

plot(Pv_event,'O')

ylabel ('Probability')

title ('Daily Peak Hour Fixture Probability Values');

grid on

ax=gca;

ax.XLim = [0 24];

ax.YLim = [0 0.3];

ax.XTick = (0:1:24);

% plot for running average of peak hour p-values

subplot(2,1,2)

Avg_Pv = cumsum(Pv_event')./(1:numel(Pv_event'));

plot(Avg_Pv,'-*')

xlabel ('Study Day')

ylabel ('Probability')

title ('Running Average Peak Hour Probability Values');

grid on

ax=gca;

ax.XLim = [0 24];

ax.YLim = [0 0.3];

ax.XTick = (0:1:24);

% This calculation is for data collected from multiple days at one building.

% The Suggested Fixture P-value would be the average similar fixtures in from various building

Figure 1: Summary of calculated fixture p-values

% Plots for Visualizing Daily Data (set for last study date)

ST=floor(SrtTdata(1),'minute');  % First pulse of the day

figure

subplot(2,2,1)

% plot for # Busy fixtures

plot(Time_TL,DailyTL(:,1))

hold on

plot(ST:seconds(1):ST+seconds(2),[0 2 0],'-r')    % Indicate first pulse

hold off

title ('Fixture Activation Timeline' + string(datetime(StudyDates(i))))

% title("It is currently " + string(datetime(StudyDates(i))))

ylabel ('# Busy Fixtures')

ax=gca;

ax.XLim = [duration(0,0,0) duration(24,0,0)];

ax.YLim = [-0.1 1.1];

ax.YTick = (0:1);

ax.XTick = (duration(0,0,0):duration(6,0,0):duration(24,0,0));

xtickformat('hh:mm')

grid on

% plot for Number of events per hour

subplot(2,2,2)

bar(ProbTime, Daily_PulseTL)

title ('Number of Events on ' + string(datetime(StudyDates(i))))

ylabel ('# Events')

ax=gca;

ax.XLim = [duration(0,0,0) duration(24,0,0)];

ax.YLim = [0 max(Daily_PulseTL)+2];

ax.XTick = (duration(0,0,0):duration(6,0,0):duration(24,0,0));

xtickformat('hh:mm')

grid on

% plot for claculated probability values

subplot(2,2,3)

plot(ProbTime,Daily_ProbTL,'+')

title ('Hourly Probability Values on ' + string(datetime(StudyDates(i))))

ylabel ('Probability')

ax=gca;

ax.XLim = [duration(0,0,0) duration(24,0,0)];

ax.YLim = [0 round(max(Daily_ProbTL),2)*1.2];

ax.XTick = (duration(0,0,0):duration(6,0,0):duration(24,0,0));

xtickformat('hh:mm')

grid on

% Box plot for calculated probability values

subplot(2,2,4)

boxplot(aa)

ylabel ('Probability')

title ('Boxplot of Fixture p-values on ' + string(datetime(StudyDates(i))))

ax=gca;

ax.YLim = [0 round(max(aa),2)*1.2];

ax.YTick = (0:0.01:round(max(aa),2)*1.2);

Figure 2: Summary of raw data and calculated fixture p-values on a single day (12//08/22) during the study period